Leak Detection Algorithm:

Leak detection is based on the hydraulic relationship between inflow and outflow sensors, as well as pressure gradients. The following equations govern the process:

Mass Balance Method:

Qleak = Qin – Qout. (Adedeji, 2017)

If the difference between inflow and outflow exceeds a preset threshold, a leak is suspected.

Pressure Drop Correlation:

∆P = Pentering – Pexiting

Pexiting = pressure of the water exiting the pipe. (Second sensor)

Pentering = pressure of the water entering the pipe. (first sensor)

If a significant pressure drop is associated with a flow imbalance, it strengthens leak confidence.

Leak Area Estimation (Orifice Equation):

A=QCd2∆Peρ

Where:

A: Leak Area (m2)

Q: Leak flow rate (m3/s)

Cd: Discharge coefficient (0.6-0.8)

ρ: Fluid density (kg/m3)

∆Pe: Pressure difference (Pa)

Pout = earth pressure above the leak.

Pin = pressure captured by the sensor inside the pipe after the leak.

From the estimated area, the Leak diameter is calculated as: D=2√(A/π)

Where soil density comes into play (Earth pressure)

Soil density is used to calculate external pressure on the pipe:

Psoil=ρsoil⋅g⋅h

Where:

- ρsoi​ = soil density (kg/m³)

- g = 9.81 m/s²

- h = burial depth (m)

Example

If:

- Soil density = 1800 kg/m³

- Burial depth = 2 m

Psoil=1800×9.81×2

Psoil=35,316 Pa≈0.353 bar

👉 This is external pressure acting on the leak